Noise Filtering of SEM Image [II]

$ Introduction

This application software has following features.

1. Able to make a median filtered image (8 bits bitmap or 16/8 bits tiff, with grayscale).

2. Able to make a 3D surface matched image (ditto) from the median filtered image.

$ Background of Functions

1. The kernel size of the median filter is 3x3 or 5x1.

2. The 3D surface matching is as follows:

1) A 7x7 kernel is applied to an area on an SEM image (8 bits grayscale image).

2) For the area (having 49 data points) of the kernel, a formula

Z(X,Y) = a*X*X*X + b*X*X*Y + c*X*Y*Y + d*Y*Y*Y + e*X*X +

f*X*Y + g*Y*Y + h*X + i*Y + j

is applied, and 10 coefficients of the formula are gotten by the least square approximation method.

3) A value at the kernel center point is applied to make a 3D surface matched image.

3. The high frequency enhancement is as follows:

1) The FFT (Fast Fourier Transform) is executed.

2) The following frequency filter is applied to the FFT data.

Filter = exp[ a * (w - b * w(max)) / w(max) ]  :  for  b * w(max) <= w,

= 1  :  for  b * w(max) > w,

where w is the spatial frequency and w(max) is the maximum spatial frequency.

3) Then, the inverse FFT is executed.

*************************************************************************************

Note: The random noises in an SEM image are dramatically reduced by the 3x3 median filtering.  However, artifacts by the median filter processing appear on the image.  By applying the 3D surface matching, the artifacts on the image are reduced even the image is a bit blurred.

On the other hand, the 5x1 median filtering seems to be good for an SEM image with vibration interference, because of looking less artifacts.

The high frequency enhancement seems to be not so impressive.

*************************************************************************************

$ Functions

Sets parameters.

Opens an SEM image file (8 bits bitmap or 16/8 bits tiff, with grayscale).

Execute the median filtering to the opened image.

Executes the 3D surface matching to the image.

Executes the high frequency enhancement to the image.

Copies an image to the Clipboard.

Saves an image with a filename.

Changes the brightness and/or contrast of image.

Shows the version information.

(This file.)

Closes this application software.

$ Original Images

Regarding SEM images, prepare bitmap images with un-compressed 8 bits grayscales or tiff images with un-compressed 16/8 bits grayscales.

[SEM image size N x M:  64 <= N, M]

$ How to Use

1. Set the evaluation parameters by  button.

2. Open an SEM image (which satisfies the above requirement) by  button.

3. Make a median filtered image by  button.

4. Make a 3D surface matched image by  button.

5. Execute the high frequency enhancement by  button, if necessary.

6. Change the brightness and/or the contrast of the image by  button, if necessary.

7. Save the image by  button.

8. Copy the image to the Clipboard by  button, if necessary.

******************************************************************

Note: The image is not automatically saved.

Please use the “save button” for saving the processed image.

******************************************************************

$ Supplement

The application software NISELL95.EXE – Noise Filtering of SEM Image [I] – can process only bitmap images with 8 bits grayscales.  However, this application software NISELL98.EXE can process bitmap images with un-compressed 8 bits grayscales or tiff images with un-compressed 16/8 bits grayscales.  This is the difference between two application software.  Regarding other points, there is no difference between them.

© Kimio Kanda