Lineprofile for SEM Image [II]

Introduction:

This application software can display a lineprofile corresponding to horizontal line(s) of an SEM image (8 bits grayscale bitmap image or 16/8 bits grayscale tiff image) on the screen and save the lineprofile data to a file. (Note: An 8 bits color bitmap or tiff image can be treated, but this is an extra function. Therefore, it is not recommended to use this function.)

Usage:

1. When opening an SEM bitmap or tiff file, a horizontal bar is displayed in the SEM image.

2. If moving the mouse pointer to around the bar, the shape of mouse pointer changes to shape.

3. Then, press the left button of mouse and move the mouse pointer up/down with holding down the left button of mouse.  By this action, the bar position in the SEM image can be vertically changed.

4. When getting an appropriate bar position, release the left button of mouse.

If you want to finely adjust the vertical bar position, please press an “Up Arrow ” key or a “Down Arrow ” key under the following conditions;

1) The left/right button of mouse is not pressed, and

2) An image is displayed on the screen, and

3) The main window of this application software is activated.

Or, under the above conditions, rotate the scroll wheel of mouse , when the mouse pointer exists within the region of vertical scroll bar. (Note: This function is not supported by Windows 95.)

5. When pressing the right button of mouse on the image, a point (that indicates X coordinate of the pressed point) appears on the bar.  Then, once more pressing the right button of mouse on the image, a selected region bar (that corresponds to the two selected points) appears on the screen.

If pressing the right button of mouse one more time, the selected region bar disappears and the bar returns to the standard bar shape.

If you want to finely adjust the horizontal regional position of the selected region bar, please press a “Left Arrow ” key or a “Right Arrow ” key under the following conditions;

1) The left/right button of mouse is not pressed, and

2) An image is displayed on the screen, and

3) The main window of this application software is activated.

6. If pressing “Draw Graph” button (), you can get a lineprofile in a dedicated lineprofile window.

Note: Please use this button, when the lineprofile window is not displayed.

7. If pressing “Save Data” button () under the condition of the lineprofile window existing, you can save the lineprofile data to a file. A saved data file is a text file, but its extension is “XLS.” Because Microsoft Excel can be automatically executed, when double-clicking the file that appears in the Explorer window.

Note: Under the condition of “selected region bar,” a saved data file only contains the lineprofile data corresponding to the selected region.

Buttons:

Sets number of lines and number of pixels to be averaged for getting a lineprofile.

Shows a file selection dialog box for a file to be opened.

Shows a zoomed out displayed image. (Minimum size = at least smaller than the window size of this application software)

Shows a zoomed in displayed image . (Maximum size = 1:1)

Displays a lineprofile in the dedicated lineprofile window.

Rotates an image on the screen.

Makes a mirror image by rotating the displayed image around Y-axis.

Shows a file save dialog box for lineprofile data to be saved.

Copies the lineprofile graph to the “Clipboard.”

Copies the image with lineprofile position(s) to the “Clipboard.”

Shows a file save dialog box for the image with lineprofile position(s).

Shows “About” dialog box.

(This file.)

Closes the application.

Supplement:

The application software SEMLPF95.EXE – Lineprofile for SEM Image [I] – can process only bitmap images with 8 bits grayscales. (Note: An 8 bits color bitmap image can be treated, but this is an extra function. Therefore, it is not recommended to use this function.)  However, this application software SEMLPF98.EXE can process bitmap images with un-compressed 8 bits grayscales or tiff images with un-compressed 16/8 bits grayscales.  This is the difference between two application software.  Regarding other points, there is no difference between them.

© Kimio Kanda