8086

S2, S1, S0 26–28 O STATUS: active during T4, T1, and T2 and is returned to the passive state

(1, 1, 1) during T3 or during TW when READY is HIGH. This status is used

by the 8288 Bus Controller to generate all memory and I/O access control

signals. Any change by S2, S1, or S0 during T4 is used to indicate the

beginning of a bus cycle, and the return to the passive state in T3 or TW is

used to indicate the end of a bus cycle.

Symbol Pin No. Type Name and Function

S2, S1, S0 26–28 O These signals float to 3-state OFF in ‘‘hold acknowledge’’. These status

(Continued) lines are encoded as shown.

| S2 | S1 | S0 | Characteristics |
| --- | --- | --- | --- |
| 0 (LOW) | 0 | 0 | Interrupt Acknowledge |
| 0 | 0 | 1 | Read I/O Port |
| 0 | 1 | 0 | Write I/O Port |
| 0 | 1 | 1 | Halt |
| 1 (HIGH) | 0 | 0 | Code Access |
| 1 | 0 | 1 | Read Memory |
| 1 | 1 | 0 | Write Memory |
| 1 | 1 | 1 | Passive |

80386

5.2.4 Address Bus (BE0# through BE3#, A2 through A31)

These three-state outputs provide physical memory addresses or I/O port addresses. The address bus is capable of addressing 4 gigabytes of physical memory space (00000000H through FFFFFFFFH), and 64 kilobytes of I/O address space (00000000H through 0000FFFFH) for programmed I/O. I/O transfers automatically generated for Intel386 DX-tocoprocessor communication use I/O addresses 800000F8H through 800000FFH, so A31 HIGH in conjunction with M/IO# LOW allows simple generation of the coprocessor select signal.

The Byte Enable outputs, BE0#-BE3#, directly indicate which bytes of the 32-bit data bus are involved with the current transfer. This is most convenient for external hardware.

BE0# applies to D0-D7

BE1# applies to D8-D15

BE2# applies to D16-D23

BE3# applies to D24-D31

The number of Byte Enables asserted indicates the physical size of the operand being transferred (1, 2, 3, or 4 bytes). Refer to section 5.3.6 Operand Alignment.

When a memory write cycle or I/O write cycle is in progress, and the operand being transferred occupies only the upper 16 bits of the data bus (D16- D31), duplicate data is simultaneously presented on the corresponding lower 16-bits of the data bus (D0-D15). This duplication is performed for optimum write performance on 16-bit buses. The pattern of write data duplication is a function of the Byte Enables asserted during the write cycle. Table 5-1 lists the write data present on D0-D31, as a function of the asserted Byte Enable outputs BE0#-BE3#.

5.2.5 Bus Cycle Definition Signals(W/R#, D/C#, M/IO#, LOCK#)

These three-state outputs define the type of bus cycle being performed. W/R# distinguishes between write and read cycles. D/C# distinguishes between data and control cycles. M/IO# distinguishes between memory and I/O cycles. LOCK# distinguishes between locked and unlocked bus cycles.

The primary bus cycle definition signals are W/R#, D/C# and M/IO#, since these are the signals driven valid as the ADS# (Address Status output) is driven asserted. The LOCK# is driven valid at the same time as the first locked bus cycle begins, which due to address pipelining, could be later than ADS# is driven asserted. See 5.4.3.4 Pipelined Address.The LOCK# is negated when the READY# input terminates the last bus cycle which was locked.

Exact bus cycle definitions, as a function of W/R#, D/C#, and M/IO#, are given in Table 5-2. Note one combination of W/R#, D/C# and M/IO# is never given when ADS# is asserted (however, that combination, which is listed as ``does not occur,'' may occur during idle bus states when ADS# is not asserted).

If M/IO#, D/C#, and W/R# are qualified by ADS# asserted, then a decoding scheme may be simplified by using this definition of the ``does not occur'' combination.

Table 5-1. Write Data Duplication as a Function of BE0#-BE3#

| Intel386 Byte Enables |  |  |  | Intel386 Write Data |  |  |  | Automatic Duplication? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BE3# | BE2# | BE1# | High<- アドレス ->Low BE0# | D24-31 | D16-23 | D31<- データバス ->D0 D8-15 | D0-7 |  |
| High | High | High | Low | undef | undef | undef | A | No |
| High | High | Low | High | undef | undef | B | undef | No |
| High | Low | High | High | undef | C | undef | C | Yes |
| Low | High | High | High | D | undef | D | undef | Yes |
| High | High | Low | Low | undef | undef | B | A | No |
| High | Low | Low | High | undef | C | B | undef | No |
| Low | Low | High | High | D | C | D | C | Yes |
| High | Low | Low | Low | undef | C | B | A | No |
| Low | Low | Low | High | D | C | B | undef | No |
| Low | Low | Low | Low | D | C | B | A | No |

Key:

D e logical write data d24-d31

C e logical write data d16-d23

B e logical write data d8-d15

A e logical write data d0-d7

Table 5-2. Bus Cycle Definition

| M/IO# | D/C# | W/R# | Bus Cycle Type |  | Locked? |
| --- | --- | --- | --- | --- | --- |
| Low | Low | Low | INTERRUPT ACKNOWLEDGE |  | Yes |
| Low | Low | High | does not occur |  | - |
| Low | High | Low | I/O DATA READ |  | No |
| Low | High | High | I/O DATA WRITE |  | No |
| High | Low | Low | MEMORY CODE READ |  | No |
| High | Low | High | HALT: Address = 2 (BE0# High BE1# High BE2# Low BE3# High A2-A31 Low) | SHUTDOWN: Address = 0 (BE0# Low BE1# High BE2# High BE3# High A2-A31 Low) | No |
| High | High | Low | MEMORY DATA READ |  | Some Cycles |
| High | High | High | MEMORY DATA WRITE |  | Some Cycles |

5.2.6 Bus Control Signals (ADS#, READY#, NA#, BS16#)

5.2.6.1 INTRODUCTION

The following signals allow the processor to indicate when a bus cycle has begun, and allow other system hardware to control address pipelining, data bus width and bus cycle termination.

5.2.6.2 ADDRESS STATUS (ADS#)

This three-state output indicates that a valid bus cycle definition, and address (W/R#, D/C#, M/IO#, BE0#-BE3#, and A2-A31) is being driven at the Intel386 DX pins. It is asserted during T1 and T2P

bus states (see 5.4.3.2 Non-pipelined Address and 5.4.3.4 Pipelined Address for additional information on bus states).

5.2.6.3 TRANSFER ACKNOWLEDGE (READY#)

This input indicates the current bus cycle is complete, and the active bytes indicated by BE0#- BE3# and BS16# are accepted or provided. When READY# is sampled asserted during a read cycle or interrupt acknowledge cycle, the Intel386 DX latches the input data and terminates the cycle. When READY# is sampled asserted during a write cycle, the processor terminates the bus cycle.

READY# is ignored on the first bus state of all bus cycles, and sampled each bus state thereafter until asserted. READY# must eventually be asserted to acknowledge every bus cycle, including Halt Indication and Shutdown Indication bus cycles. When being sampled, READY must always meet setup and hold times t19 and t20 for correct operation. See all sections of 5.4 Bus Functional Description.

5.2.6.4 NEXT ADDRESS REQUEST (NA#)

This is used to request address pipelining. This input indicates the system is prepared to accept new values of BE0#-BE3#, A2-A31, W/R#, D/C# and M/IO# from the Intel386 DX even if the end of the current cycle is not being acknowledged on READY#. If this input is asserted when sampled, the next address is driven onto the bus, provided the next bus request is already pending internally. See

5.4.2 Address Pipelining and 5.4.3 Read and Write Cycles. NA# must always meet setup and hold times, t15 and t16, for correct operation.

5.2.6.5 BUS SIZE 16 (BS16#)

The BS16# feature allows the Intel386 DX to directly connect to 32-bit and 16-bit data buses. Asserting this input constrains the current bus cycle to use only the lower-order half (D0-D15) of the data bus, corresponding to BE0# and BE1#. Asserting BS16# has no additional effect if only BE0# and/or BE1# are asserted in the current cycle. However, during bus cycles asserting BE2# or BE3#, asserting BS16# will automatically cause the Intel386 DX to make adjustments for correct transfer of the upper bytes(s) using only physical data signals D0- D15.

If the operand spans both halves of the data bus and BS16# is asserted, the Intel386 DX will automatically perform another 16-bit bus cycle. BS16# must always meet setup and hold times t17 and t18 for correct operation.

Intel386 DX I/O cycles are automatically generated for coprocessor communication. Since the Intel386 DX must transfer 32-bit quantities between itself and the Intel387 DX, BS16#must not be asserted during Intel387 DX communication cycles.

5.2.7.2 BUS HOLD REQUEST (HOLD)

This input indicates some device other than the Intel386 DX requires bus mastership.

HOLD must remain asserted as long as any other device is a local bus master. HOLD is not recognized while RESET is asserted. If RESET is asserted while HOLD is asserted, RESET has priority and places the bus into an idle state, rather than the hold acknowledge (high impedance) state.

HOLD is level-sensitive and is a synchronous input. HOLD signals must always meet setup and hold times t23 and t24 for correct operation.

5.2.7.3 BUS HOLD ACKNOWLEDGE (HLDA)

Assertion of this output indicates the Intel386 DX has relinquished control of its local bus in response

to HOLD asserted, and is in the bus Hold Acknowledge state.

The Hold Acknowledge state offers near-complete signal isolation. In the Hold Acknowledge state,

HLDA is the only signal being driven by the Intel386 DX. The other output signals or bidirectional signals (D0-D31, BE0#-BE3#, A2-A31, W/R#, D/C#, M/IO#, LOCK# and ADS#) are in a high-impedance state so the requesting bus master may control them. Pullup resistors may be desired on several signals to avoid spurious activity when no bus master is driving them. See 7.2.3 Resistor Recommendations. Also, one rising edge occuring on the NMI input during Hold Acknowledge is remembered, for processing after the HOLD input is negated.

In addition to the normal usage of Hold Acknowledge with DMA controllers or master peripherals, the near-complete isolation has particular attractiveness during system test when test equipment drives the system, and in hardware-fault-tolerant applications.

5.4.4 Interrupt Acknowledge (INTA)Cycles

In response to an interrupt request on the INTR input when interrupts are enabled, the Intel386 DX performs two interrupt acknowledge cycles. These bus cycles are similar to read cycles in that bus definition signals define the type of bus activity taking place, and each cycle continues until acknowledged by READY# sampled asserted.

The state of A2 distinguishes the first and second interrupt acknowledge cycles. The byte address driven during the first interrupt acknowledge cycle is 4 (A31-A3 low, A2 high, BE3#-BE1# high, and BE0# low). The address driven during the second interrupt acknowledge cycle is 0 (A31-A2 low, BE3#-BE1# high, BE0# low).

The LOCK# output is asserted from the beginning of the first interrupt acknowledge cycle until the end of the second interrupt acknowledge cycle. Four idle bus states, Ti, are inserted by the Intel386 DX between the two interrupt acknowledge cycles, allowing for compatibility with spec TRHRL of the 8259A Interrupt Controller.

During both interrupt acknowledge cycles, D0-D31 float. No data is read at the end of the first interrupt acknowledge cycle. At the end of the second interrupt acknowledge cycle, the Intel386 DX will read an external interrupt vector from D0-D7 of the data bus. The vector indicates the specific interrupt number (from 0-255) requiring service.

5.5.1 Entering and Exiting Hold Acknowledge

The bus hold acknowledge state, Th, is entered in response to the HOLD input being asserted. In the bus hold acknowledge state, the Intel386 DX floats all output or bidirectional signals, except for HLDA.

HLDA is asserted as long as the Intel386 DX remains in the bus hold acknowledge state. In the bus

hold acknowledge state, all inputs except HOLD, RESET, BUSY#, ERROR#, and PEREQ are ignored (also up to one rising edge on NMI is remembered for processing when HOLD is no longer asserted).

Th may be entered from a bus idle state as in Figure 5-25 or after the acknowledgement of the current physical bus cycle if the LOCK# signal is not asserted, as in Figures 5-26 and 5-27. If HOLD is asserted during a locked bus cycle, the Intel386 DX may execute one unlocked bus cycle before acknowledging HOLD. If asserting BS16# requires a second 16-bit bus cycle to complete a physical operand transfer, it is performed before HOLD is acknowledged, although the bus state diagrams in Figures 5-13 and 5-20 do not indicate that detail.

Th is exited in response to the HOLD input being negated. The following state will be Ti as in Figure

5-25 if no bus request is pending. The following bus state will be T1 if a bus request is internally pending, as in Figures 5-26 and 5-27.

Th is also exited in response to RESET being asserted.

If a rising edge occurs on the edge-triggered NMI input while in Th, the event is remembered as a nonmaskable interrupt 2 and is serviced when Th is exited, unless of course, the Intel386 DX is reset before Th is exited.

バイトアドレッシング

8bit データバス

|  |  |  | リトルエンディアン |  | ビッグエンディアン |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | アドレス A0 |  |  | バス(register) |  | バス(register) |  |
| 8bit data | 0 | 0 |  |  |  |  |  |
|  | 1 | 1 |  |  |  |  |  |
| 16bit data | 0 | 0 | L | (d7-0) | H | (d15-0) |  |
|  |  | 1 | H | (d15-0) | L | (d7-0) |  |
|  | 1 | 0 1 | L | (d7-0) | H | (d15-0) |  |
|  |  |  | H | (d15-0) | L | (d7-0) |  |

BE0がアドレス下位

BE0がアドレス下位

16bit データバス

|  |  |  | リトルエンディアン |  | ビッグエンディアン |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | アドレス A0 |  | BE1-0 | バス->register 0 1 | BE1-0 | バス->register 1 0 |  |
| 8bit data | 0 | 0 | 01 | 00ff>(d7-0) | 10 | Ff00(d7-0) |  |
|  | 1 | 0 | 10 | Ff00>(d7-0) | 01 | 00ff(d7-0) |  |
| 16bit data | 0 | 0 | 11 | Ffff->(d15-0) | 11 | ffff(d15-0) |  |
|  | 1 | 0 2 | 10 | Ff00->(d7-0) | 01 | 00ff(d15-8) | 奇数アドレスエラー |
|  |  |  | 01 | 00ff->(d15-8) | 10 | Ff00 |  |
| 32bit data | 0 | 0 2 | 11 | Ffff->(d15-d0) | 11 | Ffff |  |
|  |  |  | 11 | Ffff->(d31-d16) | 11 | ffff |  |
|  | 1 | 0 2 4 | 10 | Ff00->(d7-0) | 01 | 00ff | 奇数アドレスエラー |
|  |  |  | 11 | Ffff->(d23-d16) | 11 | Ffff |  |
|  |  |  | 01 | 00ff->(d31-d24) | 10 | Ff00 |  |

32bit データバス

|  |  |  | リトルエンディアン |  | ビッグエンディアン |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | アドレス A1,0 |  | BE3-0 | バス(register) 3 2 1 0 | BE3-0 | バス(register) 0 1 2 3 |  |
| 8bit data | 0 | 0 | 0001 | 000000ff(d7-0) | 1000 | Ff000000 |  |
|  | 1 | 0 | 0010 | 0000Ff00(d7-0) | 0100 | 00ff0000 |  |
|  | 2 | 0 | 0100 | 00ff0000(d7-0) | 0010 | 0000ff00 |  |
|  | 3 | 0 | 1000 | Ff000000(d7-0) | 0001 | 000000ff |  |
| 16bit data | 0 | 0 | 0011 | 0000ffff(d15-0) | 1100 | Ffff0000 |  |
|  | 1 | 0 | 0110 | 00ffFf00(d15-0) | 0110 | 00ffff00 | 奇数アドレス |
|  | 2 | 0 | 1100 | Ffff0000(d15-0) | 0011 | 0000ffff |  |
|  | 3 | 0 4 | 1000 | ff000000(d7-0) | 0001 | 000000ff | 奇数アドレス |
|  |  |  | 0001 | 000000ff(d15-8) | 1000 | Ff000000 |  |
| 32bit data | 0 | 0 | 1111 | Ffffffff(d31-0) | 1111 | Ffffffff |  |
|  | 1 | 0 4 | 1110 | Ffffff00(d23-0) | 0111 | 00ffffff | 奇数アドレス |
|  |  |  | 0001 | 000000Ff(d31-24) | 1000 | Ff000000 |  |
|  | 2 | 0 4 | 1100 | Ffff0000(d15-0) | 0011 | 0000ffff |  |
|  |  |  | 0011 | 0000ffff(d31-16) | 1100 | Ffff0000 |  |
|  | 3 | 0 4 | 1000 | Ff000000(d7-0) | 0001 | 000000ff | 奇数アドレス |
|  |  |  | 0111 | 00ffffff(d31-8) | 1110 | Ffffff00 |  |

ワードアドレッシング

16bit データバス

|  |  |  | リトルエンディアン |  |  | ビッグエンディアン |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | アドレス |  |  | BE1-0 | バス(register) | BE1-0 | バス(register) |  |
| 16bit data | n | n |  |  | ffff |  | ffff |  |
| 32bit data | n | n n+1 |  |  | Ffff(d15-0) |  | Ffff(d31-16) |  |
|  |  |  |  |  | Ffff(d31-16) |  | Ffff(d15-0) |  |

32bit データバス

|  |  |  | リトルエンディアン |  |  | ビッグエンディアン |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | アドレス |  |  | BE1-0 | バス(register) 0 1 | BE1-0 | バス(register) 1 0 |  |
| 16bit data | 0 | 0 |  | 01 | 0000Ffff(d15-0) | 10 | Ffff0000(d15-0) |  |
|  | 1 | 0 |  | 10 | Ffff0000(d15-0) | 01 | 0000ffff(d15-0) |  |
| 32bit data | 0 | 0 |  | 11 | Ffffffff(d31-0) | 11 | Ffffffff(d31-0) |  |
|  | 1 | 0 2 |  | 01 | 0000Ffff(d15-0) | 10 | Ffff0000(d31-16) |  |
|  |  |  |  | 10 | Ffff0000(d31-16) | 01 | 0000Ffff(d15-0) |  |

| Address | 0 |  | 1 |  | 2 | 3 | 4 | 5 | 6 | 7 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ByteEnables | BE0 |  | BE1 |  | BE2 | BE3 |  |  |  |  |  |  |
|  | データバス |  |  |  |  |  |  |  |  |  |  |  |
| データバス | D0- D8- D16- D24- |  |  |  |  |  |  |  |  |  |  |  |
| Byte Data |  |  |  |  |  |  |  |  |  |  |  |  |
| Ward data |  |  |  |  |  |  |  |  |  |  |  |  |
| Dword data |  |  |  |  |  |  |  |  |  |  |  |  |

CPUの命令セット

CM_EXTERN void halWriteByte(DWORD taddr, BYTE data);

CM_EXTERN void halWriteWord(DWORD taddr, WORD data);

CM_EXTERN void halWriteDword(DWORD taddr, DWORD data);

CM_EXTERN void halOrByte(DWORD taddr, BYTE data);

CM_EXTERN void halOrWord(DWORD taddr, WORD data);

CM_EXTERN void halOrDword(DWORD taddr, DWORD data);

CM_EXTERN void halAndByte(DWORD taddr, BYTE data);

CM_EXTERN void halAndWord(DWORD taddr, WORD data);

CM_EXTERN void halAndDword(DWORD taddr, DWORD data);

CM_EXTERN void halXorByte(DWORD taddr, BYTE data);

CM_EXTERN void halXorWord(DWORD taddr, WORD data);

CM_EXTERN void halXorDword(DWORD taddr, DWORD data);

CM_EXTERN BYTE halXchgByte(DWORD taddr, BYTE data);

CM_EXTERN WORD halXchgWord(DWORD taddr, WORD data);

CM_EXTERN DWORD halXchgDword(DWORD taddr, DWORD data);

CM_EXTERN BYTE halReadByte(DWORD taddr);

CM_EXTERN WORD halReadWord(DWORD taddr);

CM_EXTERN DWORD halReadDword(DWORD taddr);

CM_EXTERN void halOutByte(DWORD tport, BYTE data);

CM_EXTERN void halOutWord(DWORD tport, WORD data);

CM_EXTERN void halOutDword(DWORD tport, DWORD data);

CM_EXTERN BYTE halInByte(DWORD tport);

CM_EXTERN WORD halInWord(DWORD tport);

CM_EXTERN DWORD halInDword(DWORD tport);

書き込みオペレーション

アドレスからバスオペレーションのBEとアドレスのセットに分解

各バスオペレーションを実行

読み込みオペレーション

アドレスからバスオペレーションのBEとアドレスのセットに分解

各バスオペレーションを実行、読み取り値からBEの部分を取り出し合成

バイトアドレッシング、リトルエンディアン

16bit bus:

A0=address & 0x01

Address &= ~0x01

Data8:

A0=0,1->BE

WriteOp(address, data, BE)

Data16:

A0=0:

WriteOp (address, data, BE=11)

A0=1:

WriteOp (address, data<<8, BE=10)

WriteOp (address+2, data>>8, BE=01)

Data32:

A0=0:

WriteOp (address, data, BE=11)

WriteOp (address+2, data>>16, BE=11)

A0=1:

WriteOp (address, data<<8, BE=10)

WriteOp (address+2, data>>8, BE=11)

WriteOp (address+4, data>>24, BE=01)

32bit bus:

A10=address & 0x03

Address &= ~0x03

Data8:

A10->BE

WriteOp (address, data, BE)

Data16:

A10=0,1,2->BE

WriteOp (address, data, BE)

A10=3:

WriteOp (address, data<<24, BE=1000)

WriteOp (address+4, data>>8, BE=0001)

Data32:

A10=0:

WriteOp (address, data, BE=1111)

A10=1->BE=1110,BE2=0001:data1=data<<8,data2=data>>24

A10=2->BE=1100,BE2=0011:data1=data<<16,data2=data>>16

A10=3->BE=1000,BE2=0111:data1=data<<24,data2=data>>8

WriteOp (address, data1, BE)

WriteOp (address+4, data2, BE2)